/*

Formal Design of Embedded Controller Using FSM

Traffic Light Controller

Arduino UNO + Proteus

*/

enum TrafficState {

STATE_NS_GREEN,

STATE_NS_YELLOW,

STATE_EW_GREEN,

STATE_EW_YELLOW

};

TrafficState currentState = STATE_NS_GREEN;

// North-South traffic signal pins

const int NS_RED_PIN    = 2;

const int NS_YELLOW_PIN = 3;

const int NS_GREEN_PIN  = 4;

// East-West traffic signal pins

const int EW_RED_PIN    = 5;

const int EW_YELLOW_PIN = 6;

const int EW_GREEN_PIN  = 7;

void allOff() {

digitalWrite(NS_RED_PIN, LOW);

digitalWrite(NS_YELLOW_PIN, LOW);

digitalWrite(NS_GREEN_PIN, LOW);

digitalWrite(EW_RED_PIN, LOW);

digitalWrite(EW_YELLOW_PIN, LOW);

digitalWrite(EW_GREEN_PIN, LOW);

}

void enterState(TrafficState state) {

allOff();

switch (state) {

case STATE_NS_GREEN:

digitalWrite(NS_GREEN_PIN, HIGH);

digitalWrite(EW_RED_PIN, HIGH);

Serial.println("S0: NS GREEN - EW RED");

break;

case STATE_NS_YELLOW:

digitalWrite(NS_YELLOW_PIN, HIGH);

digitalWrite(EW_RED_PIN, HIGH);

Serial.println("S1: NS YELLOW - EW RED");

break;

case STATE_EW_GREEN:

digitalWrite(NS_RED_PIN, HIGH);

digitalWrite(EW_GREEN_PIN, HIGH);

Serial.println("S2: NS RED - EW GREEN");

break;

case STATE_EW_YELLOW:

digitalWrite(NS_RED_PIN, HIGH);

digitalWrite(EW_YELLOW_PIN, HIGH);

Serial.println("S3: NS RED - EW YELLOW");

break;

}

}

void setup() {

pinMode(NS_RED_PIN, OUTPUT);

pinMode(NS_YELLOW_PIN, OUTPUT);

pinMode(NS_GREEN_PIN, OUTPUT);

pinMode(EW_RED_PIN, OUTPUT);

pinMode(EW_YELLOW_PIN, OUTPUT);

pinMode(EW_GREEN_PIN, OUTPUT);

Serial.begin(9600);

enterState(currentState);

}

void loop() {

switch (currentState) {

case STATE_NS_GREEN:

delay(10000);  // 10 seconds

currentState = STATE_NS_YELLOW;

break;

case STATE_NS_YELLOW:

enterState(STATE_NS_YELLOW);

delay(3000);   // 3 seconds

currentState = STATE_EW_GREEN;

break;

case STATE_EW_GREEN:

enterState(STATE_EW_GREEN);

delay(10000);  // 10 seconds

currentState = STATE_EW_YELLOW;

break;

case STATE_EW_YELLOW:

enterState(STATE_EW_YELLOW);

delay(3000);   // 3 seconds

currentState = STATE_NS_GREEN;

enterState(currentState);

break;

}

}